# DD MMMM YYYY

TRANSMITTED ELECTRONICALLY

Kelly T. Walsh

CNA Program Manager

Canadian Numbering Administrator (CNA)

COMsolve Inc.

880 Taylor Creek Drive, Room 102

Orleans, ON K4A 0Z9

# Subject:	CSCN Direction to Canadian Numbering Administrator (CNA) re: the 2027 Numbering Resource Utilization Forecast (2027 NRUF) Methodology and Assumptions

On DD MMMM YYYY, the Canadian Steering Committee on Numbering (CSCN) discussed and agreed to the direction for the CNA with respect to the 2027 NRUF Methodology and Assumptions.

The attached document contains the direction titled "CSCN Direction to CNA re: the 2027 NRUF Methodology and Assumptions, DD MMMM YYYY".

Sincerely,

# Original signed by

Kelly T. Walsh

CSCN Chair

c.c.:	Suneil Kanjeekal – CRTC staff

Étienne Robelin – CRTC staff

Alexander Pittman – CRTC staff

Attachment

CSCN Direction to CNA re: the 2027 NRUF Methodology and Assumptions

DD MMMM YYYY

The CSCN submits the following methodology and assumptions to the CNA for the 2027 Numbering Resource Utilization Forecast (NRUF).

- If there is a discrepancy between the CNA records and those submitted by the Code/Block Holder with respect to the quantities of actual Codes/Blocks assigned and reserved as of 1 January 2027, the CNA will attempt to rectify the discrepancy. However, if the discrepancy cannot be resolved, the quantity of Codes/Blocks appearing in the CNA’s records will be used. The Code/Block Holder and the CNA should attempt to resolve the discrepancy before the next NRUF is conducted.

This problem generally occurs when a CO Code:

- is a Test Code (i.e., legacy, NPA Relief, industry plant test codes and Appendix D temporary plant test codes or Thousands Block Pooling production test Code/Block); or

- has been assigned and a Part 4 Form has not been received. In the past some Code Holders have not counted codes in the Assigned state.

- CRTC staff instructed the CNA to reserve a number of CO Codes to be used for new unknown entrants, new technologies and other unforecast demand. The CSCN recommends that the quantities identified by CRTC staff should be carried forward to the 2027 NRUF, except in NPAs where pools of CO Codes have been established for initial CO Code assignments, in which case the allowance for unforecast demand should only be included for forecast years following the dissolution of the pool for initial CO Code assignments as noted in the table below.

| CRTC Staff Allowance for Unforecast Demand based on CRTC staff letter, dated 16 Oct 2007 (http://cnac.ca/NRUF/NRUF.htm) |  |
| --- | --- |
| NPA | Quantity of CO Codes |
| 204/431 | 3 |
| 226/519/548 | 5 |
| 236/250/604/672/778 | 7 |
| 249/705 | 5 |
| 289/365/905 | 7 |
| 306/639 | 3 |
| 343/613 | 7 |
| 367/418/581 | 3 |
| 403/587/780/825 | 7 |
| 416/437/647 | 6 |
| 438/514 | 6 |
| 450/579 | 5 |
| 506 | 3 |
| 709 | 2 |
| 782/902 | 3 |
| 807 | 2 |
| 819/873 | 2 |
| 867 | 2 |

Where a Notice of Consultation (NoC) is currently in effect in an NPA complex, the number of Codes/Blocks listed under “Quantity of Codes/Blocks” in the table above is superseded by any quantities specified in the related NoC. That number may be further impacted by recent Code/Block assignments from the new entrant pools.

| CRTC Staff Allowance for Unforecast Demand based on NoC |  |  |  |  |
| --- | --- | --- | --- | --- |
| NPA | Quantity of CO Codes | Relief year (est. = estimated) | Allowance to be excluded from forecast total quantities prior to the year below (= year after the dissolution of the pool, which is 2 years after relief) | CRTC Telecom Decision or Notice establishing pool of CO Codes for initial CO Code assignments |
| There are currently no NPA Complexes with numbers set aside for Unforecast Demand based on an NoC |  |  |  |  |

The quantities of Codes/Blocks in the above tables should be carried forward for the 20-year study period with no growth.

- Where the CRTC has ordered or an RPC has recommended that quantities of CO Codes be set aside for a specified period of time for assignment to initial CO Code Applicants for a 2-year period after implementation of an Overlay, the CNA shall add such quantities to the actual quantity of CO Codes for 1 January of the current year and carry them forward in the forecasts until the Relief Date, since these set-aside CO Codes are unassignable from the date of the Decision until immediately prior to the Relief Date, after which they become assignable (with limitations). The CNA should exclude such set-aside CO Codes from the calculation of annual growth rates.

- Future projections beyond the six-year forecast period will be calculated with linear extrapolation using one of the following three values:

    - Average five-year historical growth; or

    - Average of all six years of forecasted growth; or

    - Average of last five of the six forecasted years of growth.

Typically the CNA should choose the lowest average of the 3, which has generally been the average five-year historical growth. Where the CNA believes, based on its analysis, that the average five-year historical growth may not be the best methodology for that NPA for projecting growth beyond the six-year forecast period, the CNA will select one of the other two averages as appropriate. In all cases, the CNA will review the results and their projection methods with CRTC staff, who will approve the results for publishing.

- The CNA shall provide for each NPA the total quantity of actual and forecast Codes/Blocks and a breakdown of the quantity of “Unassignable CO Codes” as per section 3.7 of the CRTC-approved Thousands-Block (NXX-X) and Central Office Code (NXX) Assignment Guideline (Canadian TBCOCAG), or as otherwise directed in writing by CRTC staff when the draft aggregate results are released.

- The “Administrative Codes” and “Stranded CO Codes” shall not be used in the calculation of the average annual future growth used for the 7- to 20-year projection.

- The CNA shall not add or include any demand for Codes/Blocks for proposed CLECs that did not submit NRUF forecasts, other than the demand that is already allowed for in the quantity of Codes/Blocks for unforecast demand specified by CRTC staff.

- For the purpose of the NRUF the CNA should assume that the Overlay Method will be used for future NPA Reliefs unless CRTC staff advises otherwise.

- With respect to NPAs that are due to exhaust approximately in the 2047 timeframe, the CNA should exercise its best judgment in finalizing the forecast for those NPAs.